AMF 82-70 5 BOARD CHASSIS

ELECTRICAL PROBLEMS

CAUSES & CURES

The following is not a complete list, however, it does include the most common problems and their most common cures.

Don’t mix & match PCB’s. Once you have a chassis that works, mark it AND the PCB’s with that lane # & keep them together. They only play nice with their closest friends.

Any time you have a problem with a board, clean the contacts (where it slides into the mother board) with a pencil eraser. This may be the only thing wrong with the board.

Most of the problems can be caused by several different things, on more than 1 PCB. With the exception of the items listed below, change the #1 PCB 1st, #5 PCB 2nd & #3 PCB 3rd.

Almost all pindication problems will be caused by the #4 PCB but can also be caused by the #3 PCB.

X-Locks (strike locks) – during X cycle, acts like it’s waiting for a 9 pin but there’s already one there) are most commonly caused by the #5 PCB.

Table or sweep chatter are most commonly caused by the #3 PCB or the heat sink assy located under the large transformer.

Cycling when turned on is most commonly caused by the AUX PCB. The small one screwed to the side of the chassis.

# PCB #1

Primary function: Translates machine movement into electrical signals that the chassis can understand & visa versa, through the cam switches & other switches throughout the machine.

| ## PROBLEM | ## REPLACE COMPONENT(S) |
| --- | --- |
| No Cycle | CYCLE neon |
| No Table Spot | BS neon, T9, SB circuit, SP relay |
| Not switching ball cycles or switching by itself | Zero neon, BS or Foul circuit |
| Foul light malfunction (of all kinds) | T1, photocell |
| OS machine goes to 2nd ball then sweeps pins | T8 |
| No cycle control (flip-flop at control box) | ZERO photocell, OS photocell, T7, T8, T9, T12, T16 |
| No sweep run thru after 66º | T5, T6 |
| No sweep stop at 66º | T10, T11, SB circuit |
| Sweep Drops To 66º, Then Nothing | T12 – T15 |
| Cycles when turned on | T16 – Also check AUX PCB |
| Continuous sweep or cycle when turned on | T15 |
| Stays in 1st or 2nd ball / sweeps off spare | SA neon, T12 – T15, TA2 circuit |
| Respots (flip-flop to 2nd ball) | T7, T8 |
| Interlock during spotting | ZERO neon, T10, T11 |
| Continuous sweep | SA photocell, T5 |
| Spots on 1st ball | T4 |
| X-Locks | T9, Add Capacitor |
| Time delay too short | C1, T13 in time delay circuit, T11 in B circuit, C2 |
| Table Chatter | BS neon, T9 |
| No instructomat | Lamp, D2 |
| Interlock - Sweep & table start at same time after time delay on 1st ball | T5, T10, T12, T16, SB OR TA2 Circuits |
| No X cycle | T3 |
| No sweep stop at 270º | SA circuit |

Note: If component identified as photocell, neon or lamp, check both the light & the photocell.

# PCB #2

Primary function: Logic circuits

| ## PROBLEM | REPLACE COMPONENT(S) |
| --- | --- |
| No strike | T20, T21, T22, D3 |
| False Strike | T21, T22 |
| No spot | T25, T26 |
| Continuous table | T28 |
| Continuous table / Flashing pindication | T4 |
| Continuous sweep | T2, T13, T19 |
| Continuous cycle | T26 |
| 2nd ball light on with 1st ball light | T1 |
| False foul | T24 |
| No sweep – False cycle – No pindication | T4 |
| Table stops at about 90º | T6 |
| Stays in 1st ball | T18 |
| Interlock – Table & sweep start at same time – 1st ball | T4, T5 |
| No cycle | T7, T12 |
| No 2nd ball light | T1 |
| Table chatter | T11 |
| No cycle flip-flop from control box | T7 |

# PCB # 3

Primary function: Table, sweep & pindication driver control. Also, ball cycle retentive memory.

| ## PROBLEM | REPLACE COMPONENT(S) |
| --- | --- |
| Table chatter | TR1 & 100 ohm resistor, T5 |
| Sweep chatter | TR2, R26, T6 |
| No 1st ball light | R34, T10, T12 |
| No 2nd ball light | R35, T11, T13 |
| Stays in 1st or 2nd ball | Magnetic relay, T1, T2, T3, T15, D21 |
| No flip-flop from 2nd to 1st | T16, T17 |
| Jumps from 1st to 2nd ball | T6 |
| Interlocks | TR1, TR2, T6 |
| Flashing pindication | R28, R56, T7 |
| Continuous foul | T8, R28, SCR1 |
| No 1st ball feel for pins – Continuous spot – Spot on sweep | T3, T4 |
| No spot | T3, T4, D3 |
| Sweep stall at 180º | ? T6 ? |

# PCB #4

Primary function: Pindication

| ## PROBLEM | REPLACE COMPONENT(S) |
| --- | --- |
| No individual pindication | SCR (pin #) |
| Continuous (weak) 2nd ball light | SCR11 |
| Continuous 1st ball light | SCR12 |
| No X cycle | T1 – T10 or D1 – D10 |

# PCB#5

Primary function: 10 & -6.8 VDC power supply. Also includes 2 logic circuits: INPUT G & NOR 5 for the time delay memory circuit. Neither of which go bad often.

| PROBLEM | REPLACE COMPONENT(S) |
| --- | --- |
| X-locks – Fingers closed – Continuous table or sweep | T3, T6, T7, T8, adjust potentiometer |
| 10 VDC at 12.7 or higher | R14, T6 |
| 10 VDC drops under load | T6 |
| -6.8 VDC low or too high | Z1, C1, D1 – D4 |
| Stays in 1st ball | T8 |
| Feels for pins on 2nd ball cycle | T1 |
| Spots pins on 1st ball cycle | T7 |
| Continuous sweep | Z1, T5 |
| No cycle – No 10 VDC | R14, F2, D8, D9, Sand paint off heat sink under T6 |
| Blowing F1 | D1 – D4 |
| Blowing F2 | D7 – D10 |
| Ball stepper button causes cycle | C1 |
| Intermittent miscellaneous problems | Fuse clips loose |

# MISCELLANEOUS

| PROBLEM | REPLACE COMPONENT(S) |
| --- | --- |
| Continuous X cycle | SP relay |
| Fingers closed (no 1st ball feel for pins) | SP relay |
| No spot | SP relay or socket |
| Cycles when turned on | Aux board |
| Stays in 1st or 2nd ball | Aux board – T1, T2 |
| Sweep & table out of time (interlocks) | Aux board – T3, T4 |
| Continuous sweep or table when turned on | Aux board – T3 |
| No 150 VDC | CP3, D2 |
| Blowing CSM-1 or CSM-2 | Sweep contacts broken or loose |
| Blowing CTM-1 or CTM-2 | Table contacts broken or loose |
| Blowing F1 (M & M2 relays may chatter) | Heat sink assy |
| Table or sweep chatter | Heat sink assy component bad or something grounding it to chassis frame, loose mother board ground, BS or BS wires |
| Sweep pauses or stops at about 300º | OS switch or circuit |
| ## For Accuscore II |  |
| No control box or console cycle (RPO) | EMC board – K1, K2, U11, U15, U17 |
| Continuous cycle (10th frame sweep after spare) | EMC board – K1, K2, U11, U15, U17 |